A Systematic Synthesis for High-OrderSquare-Root Domain Filters with Reduced Voltage

Gwo-Jeng Yu

Department of Computer Science and information Engineering, Cheng Shiu University Niaosong, Taiwan 833, R. O. China. gjyu@csu.edu.tw

Received February 2, 2008

Abstract

A systematic synthesis for high-order square-root domain filter (SRD) based on the quadratic I-V relationship for an MOSFET operated in saturation region is presented in this paper. Emphases are placed on the methodology of filter synthesis, the constructive settings of DC components for input signals, the DC voltages of the state-space variables, and the DC bias current I0. The proposed prototypes of square-root domain filters are able to overcome the possible inequality between the input and output node of DC level, in which improve the reliability of high-order filter implementation. Furthermore, by means of adjusting the range of the DC bias current I0 in the acceptable boundary, the center frequency f0 or 3 dB frequency f3dB of the proposed prototypical circuits of SRD filters are not only attainable at megahertz frequencies but also tunable electronically. Simulations are performed with the model of a 0.25 m CMOS process at 1.5 V supply voltage. The simulated results, which provide that the average errors of frequency response are smaller than 1.0%, demonstrate the validity of the proposed synthetic technique. The synthesized filters have the features of high frequency operation, tuneability, extensibility, and low power consumption. [Life Science Journal. 2010; 7(1): 15-29] (ISSN: 1097 – 8135)

Key Words: high-order square-root domain filter; low-voltage level-shift current mirror; current-mode square-root circuit; electronically tunable; high frequency operation; extensibility

Introduction

Today, many researchers have dedicated to developing companding filters which comprise log-domain and square-root domain (SRD) filters. Log-domain filter which was originally proposed by Adams[1] is a nonlinear (exponential) mapping on the state variables of a state-space (SS) description of a linear transfer function and includes the characteristics of high-frequency operation and tuneability. Frey[2-3] further utilized the exponential transformation of the state-space description of a linear transfer function, in terms of bipolar circuits, to implement log-domain filters and in turn, was embedded in as forming part of a broader branch of structures by Tsividis[4]. Furthermore, Drakakis et al.[5-7] proposed a systematic synthesis method, based on “Bernoulli cell”, to realize the log-domain filters.

Most of the IC fabrication technologies, however, trend to digital CMOS processes, and this is due to the fact that a similar I–V exponential relationship holds the concept of “log-domain filtering” has been extended to the MOS transistors in weak-inversion region. Thus, adopting a direct transformation of the corresponding implementations based on bipolar transistors into MOS transistors realizations. To meet such demand and change, Toumazou et al.[8]and Germanovix et al.[9] proposed log-domain filters in terms of MOSFET circuits that operate in weak inversion. The main drawbacks of these topologies are the increased effect of transistor mismatches and the limited operation frequency, both originated from the operation mode of MOS transistor. As a result, the problems of the above methods are solved and enhanced via adopting the quadratic I–V relationship for the MOS transistor that operates in saturation region to implement log-domain filters-reducing fabrication cost while also lessening drawbacks such as high power supply voltage, high power consumption and low operation frequency. The methods are according to those of Eskiyerli et al.[10, 14], Yu et al.[11-12] and Lopez-Martin[13]. For this reason, these filters are also known as square-root domain filters. The basic building blocks for these filters include current mirrors, current-mode geometric-mean and multiplier-divider circuits. Different topologies for the geometric-mean circuits based on stacked MOS translinear loop[14-17]and up-down MOS translinear loop [18-19] are available in the open literature. Furthermore, both geometric mean and squarer blocks are used to implement the current-mode multiplier-divider[18-19].

For reliable operation of submicron digital CMOS integrated circuits, the continuous supply voltage decrease has become notable. Some current mirrors (CMs) are currently in use in most applications[20-22], whereas most of CMs either reveal severe performance degradation or are not functional at all in a low voltage supply environment. All high impedance CMs proposed by Mulder et al.[23], Zeki et al.[24] and Blalock et al.[25] require high input voltage, which provide the capability of high output voltage signal swing. The CMs proposed by[26-28] operate with low input voltage, whereas the main disadvantage of Prodanov et al.[26] has limited current range, Heim et al.[27] is not suitable for high frequency applications, and partial transistors of Itakura et al.[28] are biased in triode mode such that the mirror is sensitive to geometry and threshold voltage mismatches.

Hence, Lopez-Martin et al.[29-30] proposed 1.5 V CMOS square-root domain low-pass and companding filters. The operation frequencies, nonetheless, remain low. In Yu et al.[31-32], an experiment based on 1.5 V square-root domain band-pass filter is proposed based on the MOSFET square law which contains the following characteristics and advantages: low cost, high frequency operation, low supply voltage operation, low power consumption, low noise, and electronically tunable of center frequency.

In this paper, based on the proposed prototypical circuits of SRD filters, a systematic synthesis method of high-order SRD filters is presented. Furthermore, in order to verify the extensibility of high-order filters, the synthetic procedures are also presented and discussed in which principles must be paid attention to at time of the syntheses for high-order filters, for example, the aspect ratios W/L of transistors, capacitance C, quality factor Q, DC bias voltages of driving MOSs, and DC bias current, etc. The main objectives are not only the demonstration of practicability of high-order low-voltage SRD filters, but also indicate the design matter requiring attention of the high-order filters.

Simultaneously, the simulated results of syntheses for several high-order low-voltage SRD filters are performed and compared with the specified values in which demonstrate the validity of the proposed synthetic technique and the high performances of the proposed prototypes of SRD filters.

Design Methodologies of Prototypes for Square-Root Domain filters

Square-root domain (SRD) filters feature a nonlinear (exponential) mapping on the state variables of a state-space description for a particular transfer function. In order to implement the filters, state equations must be further transformed to nodal equations at the nodes of grounded capacitors. Furthermore, based on the MOSFET square law relationship, the nodal equations are replaced by intermediate variables. Then, by means of interconnection of sub-circuits to realize the terms of nodal equation, the design procedures of square-root domain filters are achieved.

First, the transfer function of a band-pass filter (BPF) is introduced and shows how it is used to yield a SRD BPF. Assume that the transfer function of a second-order SRD BPF is expressed as

H(s)=ω0Qss2+ω0Qs+ω02  				(1)

and by using the standard technique[33-35] for creating companion-form dynamical equations, Eq. (1) is realized by the system described by the following equations.

x1=−ω0x2x2=ω0x1−ω0Qx2+ω0Qu			(2)

where x1, x2, y, and u are state variables, output and input signals, respectively. Let the node voltages V1 and V2 be the state variables x1 and x2, and a voltage signal U denotes the input u, then (2) is rewritten as follows

CV1=−Cω0V2CV2=Cω0V1−(Cω0Q)V2+(Cω0Q)U	(3)

with C is a multiplying factor. Both of CV1andCV2in (3) are regarded as the time-dependent current through the two capacitors C connected between V1 and ground and between V2 and ground, respectively.

Assume that the drain current of an MOSFET transistor operated in saturation region is expressed as

ID=μ0CoxW2LVGS−VT2=βVGS−VT2	(4)

where β, VGS, and VT are the device transconductance parameter, the gate-to-source voltage, and the threshold voltage, respectively.

Hence, based on (4), the state equations of Eq.(3) are transformed into [11-12, 31-32]

CV1=−I0I2−ITCV2=I0I1−I0I2Q+I0IUQ+IT	(5)

where

I1≡βV1−VT2			          	(6)

I2≡βV2−VT2			          	(7)

IU≡βU−VT2			              (8)

IT≡Cω0VT 				              (9)

and

I0≡C2ω02β  or  f0=ω02π=βI02πC (10)

Note that (6)-(10) are defined as the currents in the circuit corresponding to input voltages V1, V2, U, the DC compensation term of the threshold voltage VT and the DC bias current, respectively. The SRD BPF is driven by the external applied input voltage U and DC bias current I0. Hence, based on the given device parameters β and VT of the transistors, the capacitance C, the DC bias current I0, and the quality factor Q, the center frequency f0 and the DC compensation current IT can be calculated. Moreover, the current IU is also yielded to correspond to the input voltage U. However, the currents I1 and I2 are changed according to the variations of the node voltages V1 and V2. The output is then taken from the node voltage V2.

The center frequency f0, defined in (10), is proportional to the square root of the DC bias current I0, whereas is inversely proportional to the capacitance C. Therefore, the center frequency f0 is tuned by using the capacitance C and the DC bias current I0.

Similar to the above methodologies, the equivalent representations for first-order and second-order SRD filters may be derived as listed in Table 1.

Next, in order to achieve low voltage circuit operation, both of the n-type and n-type low-voltage (LV) current mirrors (CMs) in the proposed SRD filters are designed by means of LV level-shift technique, as shown in Fig. 1[32]. Simultaneously, Fig. 2[32] shows the DC biasing circuit diagrams of the proposed n-type and p-type LV level-shift CMs, respectively, where VN1, VN2, VP1, and VP2 are connected to the corresponding bias terminals of the CMs, such that the overall supply voltage source is simplified to be only one supply voltage source VDD.

Table 1.  Equivalent representations of the SRD filters.

| Types of filters | Transfer functions | State equations | Equivalent equations of KCL |
| --- | --- | --- | --- |
| Second-order band-pass filter | H(s)=ω0Qss2+ω0Qs+ω02 | x1=−ω0x2x2=ω0x1−ω0Qx2+ω0Qu | CV1=−I0I2−ITCV2=I0I1−I0I2Q+I0IUQ+IT |
| First-order low-pass filter | H(s)=ω0s+ω0 | x=−ω0x+ω0u | CV1=−I0I1+I0IU |
| Second-order low-pass filter | H(s)=ω02s2+ω0Qs+ω02 | x1=−ω0x2+ω0ux2=ω0x1−ω0Qx2 | CV1=−I0I2+I0IUCV2=I0I1−I0I2Q+1−1QIT |
| Second-order biquad filter | H1(s)=yu1 =ω02s2+ω0Qs+ω02H2(s)=yu2 =ω0Qss2+ω0Qs+ω02 | x1=−ω0x2+ω0u1x2=ω0x1−ω0Qx2+ω0Qu2 | CV1=−I0I2+I0IU1CV2=I0I1−I0I2Q+I0IU2Q+IT |

M4

≡

M3

M7

VP2

Fig. 1.  (a) Symbol and circuit diagram of the n-type LV level-shift CM, (b) Symbol and circuit diagram of the p-type LV level-shift CM.

Fig. 2.  DC biasing circuit diagram of (a) n-type and (b) p-type LV level-shift CMs.

Note that, in Fig. 1(a), the transistor M5 biased by a voltage VN1 is acted as a current source. The VDS voltage of transistor M1 is fixed to a constant value by transistor M4, current source transistor M5, and voltage source VN2. The transistor M4 with bias voltage VN2 provides shunt feedback to form lower input impedance, which reduces the input voltage requirement.

≡

Further, according to the MOSFET square law, the square-root of two currents is realized by the current-mode square-root circuit as shown in Fig. 3[32]. The core circuit is composed of transistors M31, M32, M33, and M34. The other transistors except M31-M34, which are the LV level-shift CMs, are used to transmit current signals. Assume that the aspect ratios of the transistors in core circuit satisfy the following constraints:

β31=β32≡β  and β33=β34≡2β 	(11)

Thus, from the derivations of[31-32], we have

Iout=IxIy 		              	(12)

which conforms to the function of the current-mode square-root circuit.



Fig. 3. (a) Symbol of the current-mode square-root circuit, (b) circuit diagram of the current-mode square-root circuit.







Fig. 4.  Prototypical circuit of the proposed second-order LV SRD BPF.





Fig. 5.  Prototypical circuit of the proposed first-order LV SRD LPF.







Fig. 6.  Prototypical circuit of the proposed second-order LV SRD LPF.









Fig. 7.  Prototypical circuit of the proposed second-order LV SRD biquad.

Obviously, the constraint for the minimum power supply voltage VDD is determined by following conditions,

VDD≥2VEFP+VEFN	                (13)

VDD≥2VEFP+VTP	                (14)

VN1=VDD+((WL)61(WL)62+(WL)61(WL)63−2)VTN((WL)61(WL)62+(WL)61(WL)63+1)	 (15)

VN2=VN1+VTN+((WL)61(WL)62)VN1−VTN	 (16)

where VTN and VTP are the threshold voltage of the NMOS and PMOS transistor, respectively, VEFN≡VGSN−VTN and VEFP≡VSGP−VTP are defined as the minimum source-to-drain voltages required to sustain an NMOS transistor and PMOS transistor in saturation, respectively.

Hence, the prototypical circuit of the proposed second-order LV SRD BPF expressed as (5) is realized by using the LV level-shift CMs, three current-mode square-root circuit blocks, and two capacitors, as shown in Fig. 4[32], where U and I0 are the external applied DC biasing input voltage and DC biasing current, respectively, and V2 is the desired output voltage. Furthermore the currents IU, I1, and I2 in the BPF are related to the corresponding input voltages U, V1, and V2, respectively. The DC bias current I0 is used to control the position of center frequency of this BPF. As evident, the BPF having the expected transfer function may operate at lower supply voltage.

Similar to the above technologies, the prototypical circuits for proposed first-order LV SRD LPF, second-order LV SRD LPF, and second-order LV SRD biquad may be given, which are shown in Figs. 5, 6, and 7[32], respectively. Simultaneously, Figs. 1 to 7 are regarded as prototypes (or basic block diagrams) while carrying out the syntheses of high-order LV SRD filters.

Furthermore, consider the proposed circuits based on TSMC 0.25 m 1p5m CMOS process with VTN=0.53 V and VTP=0.58 Vto verify the functions of the proposed second-order LV SRD BPF. The maximum operation current is 200 A, the power supply voltage VDD for the proposed LV SRD BPF as shown in Fig. 4 is set as 1.5 V, and two capacitors in this BPF are C1 = C2 = 0.9 pF. The DC biasing voltages of VN1, VN2, VP1, and VP2 under VDD = 1.5 V are measured to be 0.52 V, 1.02 V, 0.95 V and 0.46 V, respectively. The microphotograph of the proposed second-order LV SRD BPF is shown in Fig. 8 where the total area of this filter is 0.0175 mm2.

Table 2 shows the measured and simulated results for the proposed LV SRD BPF and LPF, respectively; Table 3 shows the simulated results for the proposed LV SRD biquad filter[32]. Both of the measured and simulated results for the proposed LV SRD BPF are in well agreement and demonstrate that the center frequency f0 are tunable electronically, further indicate that the proposed circuit is able to provide reliable operation at 1.5 V with low power consumption and high noise immunity.

Systematic Synthesis for High-Order Square-Root Domain Filters

Assume that the circuit diagrams of LV current-mode level-shift CMs, square-root circuit, the first-order and second-order LV SRD filters shown in Figs. 1 to 7 are regarded as the prototypes (or basic block diagrams), then the transfer function of a high-order LV SRD filter can be decomposed into the product of several first-order and second-order ones, which can be implemented by cascading the corresponding prototypes.

Note that every individual pole frequency of the first-order or the second-order LV SRD filter must be adjusted to meet the specified pole frequency. Simultaneously, for realizing the high-order filter, the possible inequality between the input, output node, and grounded capacitors of DC level which will induce the incomplete power propagation and seriously degrade the frequency response of the filter all must be improved. Hence, in the course of designing, additional condition will be joined by setting the DC components of input signals and state-space variables to be a same DC voltage value, in order to guarantee that the DC levels of the input node, the output node, and the grounded capacitors are equal. Fig. 9 shows the synthetic procedures of high-order LV SRD filters in which the aspect ratios W/L of transistors for the current-mode CMs and square-root circuit shown in Table 4 are assumed to be fixed, while those of transistors and C for the current-mode LV SRD filters shown in Table 5 are allowed to be adjustable such that the DC bias current I0 may fall into the acceptable boundary which are demonstrated in the simulated and measured results of[32].

100 m

175 m

Fig. 8.  The microphotograph of the proposed second-order LV SRD BPF.

Table 2.  Measured and simulated results for the proposed LV SRD BPF and LPF.

| Parameters |  | LV SRD BPF Measured results | LV SRD LPF Simulated results |  | LV SRD LPF Simulated results |
| --- | --- | --- | --- | --- | --- |
| Technology |  | TSMC 0.25 m 1p5m |  |  |  |
| Order of the filter |  | 2 | 1 |  | 2 |
| Q |  | 21.3 | 1 |  | 1 |
| C |  | 0.9 pF | 1 pF |  | 1 pF |
| Supply voltage |  | 1.5 V | 1.5 V |  | 1.5 V |
| Power consumption (average) |  | 1.598 mW | 0.366 mW |  | 0.633 mW |
| f0 or f3dB tuning range |  | f0 = 4 - 10 MHz | f3dB = 1.2 – 21.2 MHz |  | f3dB = 1.1 – 22.5 MHz |
| IM3 (Vpp = 0.1 V) |  | -44.67 dB (5.35 MHz and 5.4 MHz (in band)) | -45.3 dB (4 MHz and 5 MHz (in band)) |  | -38.6 dB (4 MHz and 5 MHz (in band)) |
| THD (Vpp = 0.1 V) |  | -26.7 dB (5.38 MHz) | -27.6 dB (1 MHz) |  | -26.0 dB (1 MHz) |
| Output noise power |  | -45.2 dBm | -46.2 dBm |  | -44.2 dBm |
| SFDR |  | 35.0 dB | 37.1 dB |  | 34.6 dB |
| Measured and simulated conditions |  |  |  |  |  |
| I0 | 90 A |  |  | 20 A | 20 A |
| f3dB f0 | 5.52 MHz 5.66 MHz |  |  | 13.7 MHz | 14.4 MHz |

Table 3.  Simulated results for the proposed second-order LV SRD biquad filter.

|  | Simulated results |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameters |  |  | Band-pass section |  | Low-pass section |
| Technology |  | TSMC 0.25 m 1p5m |  | TSMC 0.25 m 1p5m |  |
| Order of the filter |  | 2 |  | 2 |  |
| Q |  | 1 |  | 1 |  |
| Supply voltage |  | 1.5 V |  | 1.5 V |  |
| Power consumption (average) |  | 1.42 mW |  | 1.27 mW |  |
| f0 or f3dB tuning range |  | f0 = 1.9 – 17.0 MHz |  | f3dB = 1.2 – 22.2 MHz |  |
| IM3 (Vpp = 0.1 V) |  | -33.2 dB (10 MHz and 11 MHz (in band)) |  | -34.1 dB (5 MHz and 6 MHz (in band)) |  |
| THD (10 MHz,Vpp = 0.1V) |  | -25.2 dB |  | -24.8 dB |  |
| Output noise power |  | -43.7 dBm |  | -44.0 dBm |  |
| SFDR |  | 32.7 dB |  | 31.6 dB |  |
| Simulated conditions |  |  |  |  |  |
| I0 |  | 20 A |  | 20 A |  |
| f3dB1, f0, f3dB2 |  | 6.7 MHz, 10.2 MHz, 13.5 MHz |  | f3dB = 14.1 MHz |  |

Fig. 9.  The synthetic procedures of a specified high-order LV SRD filter.

Table 4.  The aspect ratios of transistors for the LV current-mode level-shift CMs and square-root circuit shown in Figs. 1 to 3

| Prototypes | The aspect ratios (W/L) |
| --- | --- |
| The proposed LV level-shift CMs (Fig. 1) | W/L = 20 m/ 2 m |
| The DC biasing circuits (Fig. 2) | (W/L)61= 2 m/ 0.25 m, (W/L)62 = 0.3 m/ 0.25 m, (W/L)63 = 0.3 m/ 0.25 m, (W/L)64 = 2 m/ 0.25 m, (W/L)65 = 4 m/ 0.25 m, (W/L)66 = 30 m/ 0.25 m |
| The proposed LV current-mode square-root circuits (Fig. 3) | W/L = 7 m/ 0.7 m except W/L = 14 m/ 0.7 m for M33 and M34 |

Table 5.  The acceptable boundaries of C, W/L, and I0 of the LV current-mode LV SRD filters shown in Figs. 4 to 7 operated at VDD = 1.5 V.

| Type of filters | Prototypes | C | W/L | I0 |
| --- | --- | --- | --- | --- |
| Second-order LV SRD BPF | Fig. 4 | 0.5 – 1.0 pF | 10 m/ 2 m - 40 m/ 2 m for M11 - M13 | 60 - 200 A |
| First-order LV SRD LPF | Fig. 5 | 0.5 – 1.0 pF | 10 m/ 2 m - 40 m/ 2 m for M76 - M77 | 2 - 40 A |
| Second-order LV SRD LPF | Fig. 6 | 0.5 – 1.0 pF | 10 m/ 2 m - 40 m/ 2 m for M71 - M73 | 2 - 40 A |
| Second-order LV Low-pass section of SRD biquad | Fig. 7 | 0.5 – 1.0 pF | 10 m/ 2 m - 40 m/ 2 m for M81 - M84 | 2 - 40 A |
| Second-order LV Band-pass section of SRD biquad | Fig. 7 | 0.5 – 1.0 pF | 10 m/ 2 m - 40 m/ 2 m for M81 - M84 | 2 - 40 A |

Simulated Results of High-Order Square-Root Domain Filters

Consider the proposed prototypical circuits based on TSMC 0.25 m 1p5m CMOS process with VTN=0.53 V and VTP=0.58 Vto verify the functions of the proposed high-order LV SRD filters.

Synthesis of Third-Order LV SRD Low-Pass Filter

For an attempt to synthesize a third-order low-voltage square-root domain low-pass filter specified in Table 6, the synthetic configurations have two types that contain one stage of second-order LV SRD LPF cascaded one stage of first-order LV SRD LPF or one stage of second-order LV low-pass section of SRD biquad cascaded one stage of first-order LV SRD LPF shown in Fig. 10(a) and (b), respectively, whereas, the synthetic methods for the two configurations are similar. Thus, accordingly, only one of the synthetic configurations, one stage of second-order LV SRD LPF cascaded one stage of first-order LV SRD LPF, is provided in the following.

Table 6.  Specifications of a third-order LV SRD LPF.

| Parameters | Specifications |
| --- | --- |
| Technology | TSMC 0.25 m 1p5m |
| Order of the filter | 3 |
| Type of filter | Low-pass filter |
| Q | 1 |
| Supply voltage | 1.5 V |
| 3 dB frequency f3dB | 1.5 MHz |

(a)

(b)

Fig. 10.  Synthetic configurations of the specified third-order LV SRD low-pass filter.

Firstly, the prototypical circuits of second-order LV SRD LPF and first-order LV SRD LPF are given in Fig. 6 and 5, respectively, which the boundaries of I0, C, and W/L are rewritten and given in Table 7. Thus, according to the synthetic procedures shown in Fig. 9, the initial values of C and W/L for the corresponding circuits are set to be C = 1 pF and W/L = 20 m/ 2 m, respectively, and let the DC components of input signal and state-space variables be U = V1 = V2 = 0.65 V. Next, the corresponding values of β, IT, and I0 defined in (4), (9), and (10), respectively, are identical for the two prototypical circuits as

β≡12μ0Cox(WL)≈30.81 V2

IT≡Cω0VT=C(2πf0)VT≈5.00 

I0≡C2ω02β=C2(2πf0)2β≈2.88 

where the DC bias current I0 falls into the acceptable boundary shown in Table 7. Hence, the simulated frequency response of third-order LV SRD LPF may be evaluated as shown in Fig. 11 in which the simulated 3 dB frequency f3dB is approximately 1.486 MHz. Obviously the error of frequency response is approximately 0.933%, while compared with the specified 3 dB frequency f3dB = 1.5 MHz

Table 7.  The acceptable boundaries of C, W/L, and I0 of second-order LV SRD LPF and first-order LV SRD LPF.

| Type of filters | Prototypes | C | W/L | I0 |
| --- | --- | --- | --- | --- |
| Second-order LV SRD LPF | Fig. 6 | 0.5 – 1.0 pF | 10 m/ 2 m - 40 m/ 2 m for M71 - M73 | 2 - 40 A |
| First-order LV SRD LPF | Fig. 5 | 0.5 – 1.0 pF | 10 m/ 2 m - 40 m/ 2m for M76 - M77 | 2 - 40 A |

Ideal curve

Simulated curve

Fig. 11.  The simulated frequency response of the third-order LV SRD LPF.

Synthesis of Eighth-Order LV SRD Band-Pass Filter

Similarly, by means of combining four stages second-order LV SRD BPF or LV band-pass section of SRD biquad cascaded, an eighth-order low-voltage square-root domain band-pass filter specified in Table 8 may be synthesized. Further, simply only the synthetic method of adopting four stages of second-order LV SRD BPF cascaded is described.

Table 8.  Specifications of an eighth-order LV SRD BPF.

| Parameters | Specifications |
| --- | --- |
| Technology | TSMC 0.25 m 1p5m |
| Order of the filter | 8 |
| Type of filter | Band-pass filter |
| Q | 1 |
| Supply voltage | 1.5 V |
| Center frequency f0 | 5 MHz |

The adopted prototype of second-order LV SRD BPF shown in Fig. 4 and Table 9 is then the corresponding boundaries of I0, C, and W/L. Furthermore, according to the synthetic procedures of Fig. 9, the initial values of C and W/L for the corresponding prototype are set to be 1 pF and 20 m/ 2 m, respectively, and assume the DC components to be U = V1 = V2 = 0.65 V. Hence, the corresponding values of β, IT, and I0 for the second-order LV SRD BPF are calculated as

β≈30.81 V2, IT≈16.65, and I0≈32.03 

Table 9.  The acceptable boundaries of C, W/L, and I0 of second-order LV SRD BPF.

| Type of filters | Prototypes | C | W/L | I0 |
| --- | --- | --- | --- | --- |
| Second-order LV SRD BPF | Fig. 4 | 0.5 – 1.0 pF | 10 m/ 2 m - 40 m/ 2 m for M11 - M13 | 60 - 200 A |

However, the DC bias current I0 is smaller than the acceptable lower limit 60 specified in Table 9. Thus, according to the synthetic procedures of Fig. 9, the aspect ratio W/L must be decreased to be W/L = 10 m/ 2 m. The resultant quantities are

β≈15.41 V2, IT≈16.65, and I0≈64.05 

which indicate DC bias current I0 satisfies the acceptable boundary specified in Table 9. Hence, the simulated frequency response of the eighth-order LV SRD BPF is performed and shown in Fig. 12 in which the simulated center frequency f0 approximates at 4.97 MHz and suffers from 0.6% error of frequency response, while compared with the specified center frequency f0 = 5 MHz.

Sixth-order

Fourth-order

Eighth-order

Second-order

Fig. 12.  The simulated frequency response of the eighth-order LV SRD BPF.

Synthesis of Ninth-Order LV SRD Low-Pass Filter

Next, in order to design a ninth-order LV SRD low-pass filter specified in Table 10, two schemes are used. Each contains four stages of second-order LV SRD LPF cascaded one stage of first-order LV SRD LPF or four stages of second-order LV low-pass section of SRD biquad cascaded one stage of first-order LV SRD LPF, respectively, are provided in below and demonstrated.

Table 10.  Specifications of a ninth-order LV SRD LPF.

| Parameters | Specifications |
| --- | --- |
| Technology | TSMC 0.25 m 1p5m |
| Order of the filter | 9 |
| Type of filter | Low-pass filter |
| Q | 1 |
| Supply voltage | 1.5 V |
| 3 dB frequency f3dB | 2 MHz |

Four Stages of Second-Order LV LPF Cascaded One Stage of First-Order LV SRD LPF. The prototypes of second-order LV SRD LPF and first-order LV SRD LPF are given in Fig. 6 and 5, respectively, and the corresponding boundaries of I0, C, and W/L are rewritten and given in Table 11. Simultaneously, the initial values of C and W/L for the corresponding prototypes are also set to be 1 pF and 20 m/ 2 m, respectively. Thus, the corresponding values of β, IT, and I0 for the two prototypes are all identical, result in

β≈30.81 V2, IT≈6.66 , and I0≈5.13 

where the DC bias current I0 meets the acceptable boundary shown in Table 11. Fig. 13 shows the simulated frequency response of the ninth-order LV SRD LPF. The simulated 3 dB frequency f3dB approximates at 2.018 MHz and results in approximately 0.9% error of frequency response.

Table 11.  The acceptable boundaries of C, W/L, and I0 of the second-order LV SRD LPF and first-order LV SRD LPF.

| Type of filters | Prototypes | C | W/L | I0 |
| --- | --- | --- | --- | --- |
| Second-order LV SRD LPF | Fig. 6 | 0.5 – 1.0 pF | m/ 2 m - 40 m/ 2 m for M71 - M73 | 2 - 40 A |
| First-order LV SRD LPF | Fig. 5 | 0.5 – 1.0 pF | 10 m/ 2 m - 40 m/ 2 m for M76 - M77 | 2 - 40 A |

Ideal curve

Simulated curve

Ideal curve

Frequency (in Hz)

Amplitude (in dB)

Fig. 13.  The simulated frequency response of the ninth-order LV SRD LPF.

Four Stages of Second-Order LV Low-Pass Section of SRD Biquad Cascaded One Stage of First-Order LV SRD LPF

Similarly, Figs. 7 and 5 are adopted as the prototypes of second-order LV low-pass section of SRD biquad and first-order LV SRD LPF, respectively. Table 12 is the corresponding boundaries of I0, C, and W/L. Next, the initial values of C and W/L for the corresponding prototypical circuits are set to be 1 pF and 20 m/ 2 m, respectively, and let the DC components be U1 = U2 = V1 = V2 = 0.65 V. Hence, the corresponding values of β, IT, and I0 for the two prototypical circuits are also identical and calculated as

β≈30.81 V2, IT≈6.66 , and I0≈5.13 

The DC bias current I0 also falls into the acceptable boundary shown in Table 12. Hence, the simulated frequency response of the ninth-order LV SRD LPF is shown in Fig. 14. The simulated 3 dB frequency f3dB approximates at 2.02 MHz and suffers from approximately 1.0% error of frequency response.

Table 12.  The acceptable boundaries of C, W/L, and I0 of the second-order LV low-pass section of SRD biquad and first-order LV SRD LPF.

| Type of filters | Prototypes | C | W/L | I0 |
| --- | --- | --- | --- | --- |
| Second-order LV low-pass section of SRD biquad | Fig. 7 | 0.5 – 1.0 pF | 10 m/ 2 m - 40 m/ 2 m for M81 - M84 | 2 - 40 A |
| First-order LV SRD LPF | Fig. 5 | 0.5 – 1.0 pF | 10 m/ 2 m - 40 m/ 2 m for M76 - M77 | 2 - 40 A |

Ideal curve

Simulated curve

Ideal curve

Frequency (in Hz)

Amplitude (in dB)

Fig. 14.  The simulated frequency response of the ninth-order LV SRD LPF.

Conclusion

Based on the MOSFET square law, a systematic synthesis method for high-order square-root domain filter with reduced voltage is presented in this paper. This work is based on the MOSFET square law. Emphases are given on synthesizing filters. The keys to this work are the constructive settings of DC components for input signals, the DC voltages of the state-space variables, and the DC bias current I0. The proposed prototypical circuits of low-voltage square-root domain filters are able to overcome the possible inequality of DC level between the input and output node. It is a significant and necessary improvement for the convenient and reliability of high-order filter realization since the inequality of input and output node of DC voltage will induce the incomplete power propagation and seriously degrade the frequency response of the filter. Moreover, by means of adjusting the range of the DC bias current I0 in the acceptable boundary, which may be established and attainable during the process of designing for the proposed prototypical circuits of SRD filters, it provides that the center frequency f0 or 3 dB frequency f3dB are not only attainable at megahertz frequencies but also tunable electronically. Simulations for the proposed high-order LV SRD filters, based on 0.25 m CMOS process and operated with 1.5 V power supply voltage, were provided and discussed throughout this paper. Simulated results, which indicate that the average errors of frequency response are less than 1.0%, demonstrate the validity of the proposed synthetic technique and the high performances of the proposed prototypical circuits of LV SRD filters. Hence, the proposed synthetic technique provides an alternative for performing high-order square-root domain filter implemented by standard digital CMOS technology.

References

- Adams RW. Filtering in the log-domain. 63rd AES Conf, New York, 1979, 1470.

- Frey DR. Log-domain filtering: an approach to current-mode filtering. IEE Proc. Pt. G, 1993;140 (12):406-416.

- Frey DR. Exponential state space filters: A generic current mode design strategy. IEEE Trans. Circuits Syst. I, 1996;43(1): 34-42.

- Tsividis Y. Externally linear, time-invariant systems and their application to companding signal processors. IEEE Trans. Circuits Syst. II, 1997;44(2): 65–85.

- Drakakis EM, Payne AJ, Toumazou C. Log-domain state-space: a systematic transistor-level approach for log-domain filtering. IEEE Trans. Circuits Syst. II, 1999;46(3):290-305.

- Drakakis EM, Payne AJ, Toumazou C. Log-domain filters, translinear circuits and the Bernoulli cell. IEEE ISCAS, Hong Kong, 1997;1:501–504.

- Drakakis EM, Payne AJ, Toumazou C. Log-domain filtering and Bernoulli cell. IEEE Trans. Circuits Syst. I, 1999;46(5):559-571.

- Toumazou C, Ngarmnil J, Lande TS. Micropower log-domain filter for electronic cochlea. Electron. Lett, 1994; 30(10):1839–1841.

- Germanovix W, O’neill G,  Toumazou C, et al. Analogue micropowered log-domain tone controller for auditory prostheses. Electron. Lett, 1998;34(11):1051-1052.

- Eskiyerli MH, Payne AJ, Toumazou C. State-space synthesis of biquads based on the MOSFET square law. IEEE Int. Symp. Circuits and Syst, 1996;1: 321-324.

- Yu GJ, Liu BD, Hsu YC, et al. ‘Design of log domain low-pass filters by MOSFET square law,’ in Proc. The Second IEEE Asia Pacific Conf. on ASICs, 2000; 9-12.

- Yu GJ, Huang CY, Chen JJ, et al. Design of Square-Root Domain Filters. Analog Integrated Circuits and Signal Processing, (Accepted), 2004.

- Lopez-Martin AJ, Carlosena A. A 3.3V tunable current-mode square-root domain biquad. IEEE Int. Symp. On Circuits and Systems, 2000;5-8.

- Eskiyerli MH, Payne AJ, Toumazou C. State-space synthesis of integrators based on the MOSFET square law. Electron. Lett, 1996;32(6):505-506.

- Mulder J, van der Woerd AC, Serdijn WA, et al.  Current-mode companding √x-domain integrator. Electronics Lett,1996;32(3):198-199.

- Mulder J, van der Woerd AC, Serdijn WA, et al. A 3.3 V current-controlled √x-domain Oscillator. Analog Integrated Circuits and Signal Processing, 1998;16(1):17-28.

- Eskiyerli MH, Payne AJ, Square-root domain filter design and performance. Analog Integrated Circuits and Signal Processing, 2000;22(2-3):231-243.

- Lopez-Martin AJ, Carlosena A. Systematic design of companding systems by component substitution.  Analog Integrated Circuits and Signal Processing, 2001;28(3):91-106.

- Lopez-Martin AJ, Carlosena A. Current-Mode Multiplier-Divider Circuits Based on the MOS Translinear Principle. Analog Integrated Circuits and Signal Processing. 2001;28(3):265-278.

- Steyaert M, Crols J, Gogaert S, et al.  Low-voltage analog CMOS filter design. IEEE Int. Symp. On Circuits and Systems, ISCAS '93, 1993;2: 1447-1450.

- Rajput SS, Jamuar SS. Low voltage, low power, high performance current mirror for portable analogue and mixed mode applications. IEE Proc. Circuits, Devices and Systems, 2001;148: 273-278.

- Ramirez-Angulo J, Carvajal RG, Torralba A, et al.    The flipped voltage follower: a useful cell for low-voltage low-power circuit design. IEEE Int. Symp. On Circuits and Systems, ISCAS 2002, 2002;3:615-618.

- Mulder J, Van der Woerd AC, Serdijn WA, et al. High-swing cascode MOS current mirror. Electronics Lett, 1996;32(14):1251 – 1252.

- Zeki A, Kuntman H. Accurate and high output impedance current mirror suitable for CMOS current output stages. Electronics Lett, 1997;33(12):1042-1043.

- Blalock BJ, Allen PE, Rincon-Mora GA, Designing 1-V op amps using standard digital CMOS technology. IEEE Trans. Circuits and Systems II,  1998;45(7):769-780.

- Heim P, Jabri MA. MOS cascode-mirror biasing circuit operating at any current level with minimal output saturation voltage. Electronics Lett, 1995;31(9):690 – 691.

- Prodanov VI, Green MM. CMOS current mirrors with reduced input and output voltage requirements. Electronics Lett, 1996;32(2):104 – 105.

- Itakura T, Czamul Z. High output-resistance CMOS current mirrors for low- voltage applications. IEICE Trans. 1997; E80-A(1):230-232.

- Lopez-Martin AJ, Carlosena A. A 1.5 V CMOS square-root domain filter. IEEE Int. Conf. On Electronics, Circuits and Systems, 2001;1465-1468.

- Lopez-Martin AJ, Carlosena A. A 1.5 V CMOS companding filter,’ Electron. Lett, 2002;38(10): 1346-1348.

- Yu GJ, Chen JJ, Lin HY, et al. A low-voltage low-power log-domain band-pass filter. IEEE Int. Symp. On VLSI Technology, Systems, and Applications, 2003;219-222.

- Yu GJ, Huang CY, Chen JJ, et al. Design of Current-Mode Square-Root Domain Band-Pass Filter with Reduced Voltage. Analog Integrated Circuits and Signal Processing, (Accepted), 2004.

- Chen CT. Linear system theory and design. (CBS College Publishing, Holt, Rinehart & Winston, New York, 1984).

- Sinha NK. Control Systems. (CBS College Publishing, Holt, Rinehart & Winston, New York, 1986).

- Nise NormanS. Control Systems Engineering.  (John Wiley & Sons, Inc., New York, 2000).