Recovering Extremely Low Frequency Signal from the Signal-Dependent Noise Background

1Hsien-Chiao Teng, 2Shen Cherng

1Department of Electrical Engineering, Chinese Military Academy, Fengshan, Kaohsiung, Taiwan 830, ROC

2Department of Electrical Engineering, Chengshiu University, Niaosong, Kaohsiung, Taiwan 833, ROC

scteng@cc.cma.edu.tw

Abstract: We developed a signal dependent noise tensor, which can be used to describe the fluctuated geomagnetic field coupled with Extremely Low Frequency (ELF) signals for our further biological signal processing study. In order to isolate the coupled ELF signals from the signal dependent noise, we introduced Quantization (QT) decoding method to discrete the noise and recover the coupled signals from the background. The signal to noise ratio of the coupling ELF can be amplified by QT in the power density spectrum (PDS). [Nature and Science. 2004;2(4) (Supplement): 1-3].

Key words: Extremely Low Frequency (ELF); noise; power density spectrum (PDS); signal

Introduction

The signal dependent noise can be presented as a noise tensor nij at time tij, in which index indicates the i th sample at energy level j. The signal can be shown as sij. Power density spectrum (PDS) analysis for tensor nij⊕sijcan be used to identify the coupled signalssijinnij. The intrinsic coupling oscillation can be captured by probe and converted to electrical voltages shown in oscilloscope.

Theory and Methods

Set an AC ELF signal sijas input to the backgroundnij, the output can be transformed to electrical voltages shown to oscilloscope.  By using HP Benchlink, we collect the output data and transform it to Microsoft Excel as text files. Matlab and Fortran programs were performed to analyze the data and get PDS. Figure 1 illustrates the flow chart of the QT process

Figure 1. The flow chart of the QT

Consider the data output sequence xij = sij⊕nij, where xij indicate the jth element in ith ensample.

Step	1: Get xij

Step	2: Set QT value from v1 to v6,  where

v1>v2>…..>v6 for six QT levels

Step	3: Computexij, the average value of xij

Step	4: If xij>xij, set mh=xij,a high threshold

valuemhshould be defined.

If xij < xij, set m1 =xij, a low threshold value m1 should be defined.

If xij > mh set mhh  = xij, a second high threshold mhh should be defined.

Step 5: Set xij = mh1 if m < xij < mh

xij = m1h, if	 ml < xij < m

xij = m11, if xij <m1, where

mhh >mh>mh1>m>m1h>m1> m11.

Step 6: If xij > mhh	, set xij = v1

If m1h<xij<mhh, set xij = v2

If mh1<xij<mh, set xij = v3

If m <xij<mh1, setxij = v4

If m1h<xij< m, setxij = v5

If	m1< xij <m1h, setxij = v6

It is defined amplitude signal to noise ratio as ASNR =AsAn , where As is the amplitude of the coupled ELF signal and An is the amplitude of the noise. In contrast, SNR =PsPn, where Ps is the power of the output ELF signal calculated from PDS and Pn is the power of the noise. Noise can be defined as all unpredictable signals in PDS. Since both ASNR and SNR can be calculated, the plot of ASNR versus SNR produces a function curve showing the correlation between input amplitude and output power.  Even through noise may have its own characteristic; we can calibrate the function curve with the help of adjusting trial signal’s amplitude to control the power difference by QT analysis. For instance, using a data sequence to simulate a sample function consisting of 2000 elements including a 15 Hz sinusoid signal, xij(t)=sij(t)+k×nij(t), label index i is from 1 to 2000 for this sequence. The dimension of the noise tensor is 2000 × j. For simplicity, take j = 1, the power spectrum can be simply calculated. Note that the identified ELF signal is supposed being occurred at 15 Hz. The frequency component of power density spectrum of the noise being illustrated will depend upon the characteristic of nij.

In addition, Figure 1b showed the magnetic fluctuation very near the cell layer on the patch substrate.

Results

The power density spectrum calculation result is illustrated in Table 1. We are not able to identify ELF 15 Hz without QT if its signal to noise ratio (S/N) is lower than 0.015.

Table 1. The power density spectrum calculation result

(- : not able to identify ELF 15Hz, however ; +: able to identify ELF 15Hz)

| ASNR | 15 Hz | QT | S/N Ratio |
| --- | --- | --- | --- |
| 1.0 | + | + | 1.5 |
| 0.5 | + | + | 0.37 |
| 0.1 | - | + | 0.015 |
| 0.05 | - | + | 0.004 |
| 0.01 | - | + | 0.0001 |

Discussion

Our results provide evidence suggesting that QT is able to affect the PDS, which is linked with the energy modulation within the noise and shows the power that the noise could sense. The purpose of this report is to provide a new method to recognize the ELF buried in signal dependent background noise.

Conclusion

The noise sensitivity to the ELF signal has been studied for a years. QT can help to recognize ELF and increase both ASNR and the SNR providing a function curve to characterize the signal dependent noise. By using this function curve, we can find the best estimate signal-to-noise ratio of the coupled ELF. The remaining question is how can we find the best combination of the weights of QT in experiments. Fuzzy and neuronet analysis may help for further noise tensor characteristic studies.

Figure 2. If S/N=1.5, the ELF signal component at 15 hertz is shown

Figure 3. If S/N=0.37, the ELF component signal at 15 hertz still can be shown Figure 4. If S/N<0.37, we are not able to identify the ELF signal at 15 hertz

Figure 5. ELF signal at 15 hertz is identified by QT when 0.0001< S/N <0.3

Correspondence to:

Hsien-Chiao Teng,

Department of Electrical Engineering, Chinese Military Academy, Fengshan, Kaohsiung,

Taiwan 830, Republic of China

,Email: scteng@cc.cma.edu.tw; Tel: 011886-7747-9510 ext 134

Reference:

frequency

- Galvanovskis J, Sandblom J. Amplification of Electromagnetic Signals by Ion Channels. Biophysical Journal 1997;73:3056-65.

- Schatzer L, Weigert S. Solvable Three-state Model of a Driven Double-Well Potential and Coherent Destruction of Tunneling. Physical Review A 1998;57(1):57-78.