Polar sea ice characteristics determined from the ENVISAT RA-2 altimeter

Lin Zhua, Lei Yangb, Dong-xu Zhoua, Xing-hua Zhoub, Qiu-hua Tangb

a Shandong University of Science and Technology, College of Geodesy and Geomatics, Qingdao 266590, China

b First Institute of Oceanography, State Oceanic Administration, Qingdao 266061, China

Tel: +86 0532-88965357; E-mail: leiyang@fio.org.cn

Abstract: The ground tracks of ENVISAT satellite cover the high latitude area that of 81.4° over north and south hemispheres, which provides a special approach to monitor sea ice using the RA-2 radar altimeter onboard the ENVISAT satellite. In this paper, we developed a method using the backscatter coefficients measured by RA-2 to detect the monthly changes of sea ice extent and surface properties over Arctic and Antarctic oceans. Based on the difference of scattering characteristics over the sea water and sea ice surface, we demonstrate that setting the RA-2 sigma0 to 13db as a threshold can separate the sea ice from sea water efficiently. Except in the summer of each hemisphere, the sea ice boundaries derived from the RA-2 altimeter and from the radiometer data by NSIDC are closely consistent. Due to the lack of measurements over central areas of Arctic, we estimated the sea ice extent of Antarctic zones only. The result of extents shows that the altimeter gives higher values in summer (southern hemisphere) compared to the radiometer, which is related to the excellent capacity of altimeter to monitor the dispersed thin ice. In cold seasons with high sea ice concentration, the disagreements of sea ice extent are very low as the mean difference is just 0.17Mkm2. Differences of polar ice properties, including surface moisture and roughness, were also studied, and result shows that the sea ice surface is dryer and rougher in Arctic than in Antarctic in the cold season.

[Lin Zhu, Lei Yang, Dong-xu Zhou, Xing-hua Zhou, Qiu-hua Tang. Polar sea ice characteristics determined from the ENVISAT RA-2 altimeter. Nat Sci 2014;12(8):40-47]. (ISSN: 1545-0740). http://www.sciencepub.net/nature. 7

Key words: backscatter coefficient, satellite altimetry, sea ice extent, sea ice boundary

# 1 Introduction

The sea ice, covering a significant fraction ( 5%-8% ) of the whole Earth’s ocean area, has remarkable seasonal variations and has a special position in the climate science and ocean ecosystem[1]. In addition, the distribution of sea ice has remarkable impacts on the navigation, oceanic constructions and resource explorations. In recent years, the polar sea ice coverage, especially over Arctic, is changing with the increasing emission of greenhouse gases that caused global climate changes [2-4]. It is predicted that the summer sea ice in Arctic ocean will be disappeared by the end of this century, which may bring tremendous influence to the polar ecosystem and the global climate[5]. The increasing importance of monitoring the sea ice extent and its properties by satellites remote sensors has long been recognized in the global climate change research. However, remote sensors at visible and infrared bands are limited over polar areas due to the polar night phenomenon and other weather reasons. The regular task of monitoring the sea ice over polar oceans relies mainly on microwave sensors, including active and passive ones.

However, most previous studies about the sea ice detection were based on data from the passive microwave sensors [6, 7]. The satellite radar altimeters , which have been widely used in the earth science[8, 9], could measure the sea surface height, significant wave height and backscatter coefficients. The ability of satellite altimeter measurements to distinguish between the sea ice and water was first researched by Dwyer and Godin[10]. Later research by Rapley from the US Seasat satellite suggested that radar altimetry can provide a powerful means of global synoptic monitoring of the interaction between ocean and ice[11]. In addition some observations have demonstrated the sensitivity of altimeter data to areas of thin ice and leads and shown the usefulness of altimetry as an adjunct to visible or infrared imagery[10]. Most of the previous study indicated that the key approach of monitoring the sea ice by the altimeter is the calculation of related parameters from the waveforms. Ulander calculated backscatter from waveforms and classed ice on the basis of backscatter[12]. Drinkwater found the theoretically inverse relationship between the integral of waveform power and ice concentration borne out in measurements[13]. Chase performed a sophisticated analysis of waveform shape, and empirically link shape to different ice types and concentrations[14]. Similar work was also done by Yang who pointed that using the correspondence between the satellite pulse altimeter waveform and reflector property and with the advantages such as the ability to make large-scale, high-resolution and long-duration observations, the altimeter can be used to determine sea ice concentration on a large scale[15]. Some researchers also combined the passive and active microwave sensors to study the sea ice. Using synergy of data radar altimeter and radiometer instruments onboard the TOPEX/Poseidon satellite Kouraev discussed time and space variations of ice extent in the Caspian and Aral seas during 1992–2002 and discussed the impact of climate change on sea ice, but the draw back was the relatively coarse cross-track coverage of T/P[16]. Previous works also showed great potential for satellite-based altimetry for estimating ice thickness and snow depth [17-19]. However, the researches mentioned above were only focused on regional area and the characteristic of sea ice distributions over the whole polar areas has not been analyzed.

In this article we will evaluate the monthly extent of sea ice in 2011 over Arctic and the sea around Antarctica by the ENVISAT radar altimeter RA-2. The sea ice extents, calculated from the altimeter, are compared to the National Snow and Ice Data Center (NSIDC) values which come from the Scanning Multi-channel Microwave Radiometer (SMMR) instrument and from Sensor Microwave Imager (SSM/I) and Special Sensor Microwave Imager/Sounder (SSMIS) instruments. For the year of 2011, the sea ice extent values of NSIDC are only provided by SSMIS. In addition to the extent, the seasonal variations of the sea ice surface moisture and roughness are also analyzed.

2 Principal and Data

2.1 Research principal

The Basic principal of the satellite radar altimeter is that it emits microwave pulses to the earth surface at the nadir point and receives the backscattered signal which then forms the altimeter waveform. From the altimeter waveforms, the sea surface height (SSH) and significant wave height (SWH) can be obtained with high precision. Besides the SSH and SWH measurements, the backscatter coefficient, directly related to scattering characteristics of reflected surface, is the key measurement for altimetry land and ice applications [20, 21]. Based on the sea ice retracking method,  at Ku band retrieved from the waveform amplitude can be expressed in [14]:

(1)

(2)

where  is a power scaling factor which is obtained at Level 1b data;is the amplitude derived by OCOG waveform tracking algorithm (Offset Centre Of Gravity); N is the number gates of the leading edge;  is the normalized power of waveform at nth gate. For more information about the waveform tracking algorithm and the sigma0 calculation readers can refer to the ENVISAT RA-2/MWR Product Handbook [22].

The backscattering coefficient results from two contributions: a surface scattering echo that is the scattering by an interface between two different media and a volume scattering echo that is the scattering by particles contained inside a medium. These two contributions depend on the target characteristics, the radar footprint (depending on surface roughness) and the quantity of surface scatterers within the footprint[23]. For the sea ice surface, beside the surface scattering there also exists volume scattering due to the scattering particles such as bubbles and salt granules inside the sea ice. For the sea water surface there is only the surface scatterings. In addition, there are also significant differences between the surface roughness of the sea water and sea ice, which will widen the scattering difference further. During the ice melting and freezing process the surface moisture and roughness will be changed accordingly, which will also influence the scattering features of the sea ice and enable us to analyze the surface properties by . Figure 1 presents an example of altimeter waveforms over the Bering Strait where is a transitional zone at the measuring time. According to the features of waveform from sea ice and water[24] we can find that in the north of 59.2°N the waveforms of red color and with high peak values are reflected by the flat sea ice. In the south of 58.9°N the waveforms, in green color lines, are typical ocean reflection. Between 59.2°N and 58.9°N the waveforms, in blue color, are generated by the mixing of sea ice and water present irregular shapes.

2.2 Research data

In this research we used ENVISAT GDR (Geophysical Data Record) from circle 98 to 110 of the year 2011 to analyze seasonal variations of the sea ice extent and surface properties. Since the sea ice is our interest, we only use  at Ku band calculated by the sea ice retracking algorithm from GDR data. ENVISAT RA-2, a new generation of satellite radar altimeter compared to T/P, was launched in 2002 and was operated by ESA. The pulse repetition frequency of RA-2 is 1795 Hz, which is corresponding to an along-track width of approximately 390m. Before October 2010 the period of ENVISAT is 35-days and the space between nearby crossovers at the equator is 80km. After October 2010 the orbit was shifted and the period was altered to 30 days. As figure 2 illustrated, compared to T/P or Jason-1/2 which only cover the earth between 66°S and 66°N, the ENVISAT radar altimeter can reach to 81.4° on both hemispheres and covers most of the polar sea area. Because the satellite covers over the southern hemisphere is similar to the northern hemisphere, so that it is not shown in figure 2. In additional, satellite ground tracks are much denser in polar areas, which is beneficial for polar ice detection. For example, the gaps between adjacent crossover points are about 40km, 30km and 15km at the latitude of 60°, 70° and 80°, respectively.

Fig.1 Typical satellite radar altimetry waveforms over the Bering Strait. The red color presents waveforms reflected by sea ice; the blue color presents waveforms reflected by the mixture of sea ice and sea water; the green color presents waveforms reflected by sea water.

The outline files of the monthly sea ice extent and the monthly extent values from NSIDC are used to valid our result. The format of outline files is the shapefile (shp) which are geospatial vector data and contain polylines of the sea ice boundary for both Northern and Southern hemispheres. The data files of the monthly sea ice extent are in ASCII text format, which are calculated from two intermediate versions of monthly gridded ice concentration fields: Sea Ice Index near-real-time monthly concentration fields and Sea Ice Index final monthly concentration fields. More information about the NSIDC data can be found at http://nsidc.org/data/docs/noaa/g02135_seaice_index.

Figure 3 shows the monthly spatial distribution of  of ENIVSAT RA-2 in 2011, which clearly demonstrates monthly changes of the sea ice cover over the whole year. The sea ice boundaries offered by NSIDC, based on passive microwave radiometers are also added to figure 3.

It is manifest that the sea ice and water can be separated clearly by  because of the significantly high values over the sea ice and relatively low values over the sea water. As the sea ice cover changes by seasons in two polar zones, the seasonal variations of the sea ice distribution can be monitored accordingly. Except the summer season (December, January and February for the southern hemisphere; June, July and August for the northern hemisphere), the altimetry and NSIDC boundaries are remarkably consistent. The primary reason to the summer diversity could be that altimeters are more sensitive than radiometers to the thin and small-scale ices which are discretely distributed on the sea in the warmer season[24]. In addition, figure 3 indicates that the differences last longer and wider in the Arctic area, which proves that there exists more discrete sea ice in warmer seasons. This phenomenon is consistent to the fact that the weather is colder in Arctic sea than Antarctica in summer.

Figure 3 also shows that over Arctic and Antarctic oceans the sea ice melts rapidly in summer. In the Northern hemisphere summer the sea ice closing to the Arctic pole can be survived due to the extremely cold temperature. However, around the Antarctica the sea ice is almost vanished except some small regional areas such as the Weddell Sea and Ross Sea. Nevertheless obviously affected by the global climate change, mainly the global climate warming, the extent of Arctic sea ice in summer frequently refreshed its minimum records and will be probably vanished in the near future according to related researches [5].

Fig. 2 Comparison of ground tracks between ENVISAT (red) and TOPEX/Poseidon, same orbits as Jason-1 and Jason-2, (green) over the northern hemisphere.

3 Polar sea ice characteristic analysis

In addition to sea ice boundaries, altimeter measurements are also used to calculate the sea ice extent around Antarctica. Data from the NSIDC are used to validate our result. Because accuracy tends to be best within the consolidated ice pack where the sea ice is relatively thick and the ice concentration is high. So we use the NSIDC sea ice outline of September, a relatively stable month for sea ice cover, to track our sigma0 grid data outside and inside the outline with different distances, and we get the average values for the track data. The statistic information indicates that sigma0 values are between 10-11db over sea water 50 km far from the sea ice boundary and sigma0 values are between 11-12db over the sea water less than 50 km away from the sea ice boundary. Along the boundary the sample result for sigma0 value is about 13db and inside the boundary the sigma0 values is about 17db over the sea ice 5km inside the boundary. So we choose 13 db as the sigma0 threshold value for calculating the sea ice extent. Figure 4 shows the comparison of monthly sea ice extents between results of the NSIDC and the altimeter. Table 1 presents the extents in detail. The average difference is 0.80Mkm2 and the standard deviation of the difference is 1.35Mkm2. The maximum of the difference is 4.42Mkm2 that appears in January while the minimum is 0.03Mkm2 that appears in April. However, the extent differences in winter (from June to September) are relatively low with the mean value of only 0.17 Mkm2. The significant difference in summer (December, January and February) is shown in fig4 which is probably caused by the sensitive capacity of altimeter and radiometer to the discretely floating sea ice. While excluding summer data the average of difference is 0.20Mkm2 and the standard deviation is 0.50Mkm2.

Fig. 3 Distribution of ENVISAT RA-2 at the Ku band over Arctic (a) and Antarctic (b) and sea ice boundaries from NSIDC (in black line) in year 2011.

In additional to the applications on the sea ice boundaries and extents, the backscatter coefficients also reflect the physical factor of the sea ice such as the surface moisture and roughness. The monthly averaged  values of the sea ice are calculated and time series are shown in figure 5. Over the Arctic the  shows a significant rise since June owing to the surface snow melting which increases the surface moisture, and the  culminates in July with the average value of more than 30db which is mainly caused by the melting water ponds on the sea ice that enhances the reflecting ability of target surface to return power to the radar altimeter. Then it decreases but still keeps high due to the relatively high surface moisture until September. After October the new ice will be formed and the sea ice distribution will be larger in Arctic which is also shown in figure 3. For the sea ice around Antarctica, however, the  values are relatively flat, which means that physical characteristics of the sea ice surface do not change significantly in the whole year. And this is consistent to the fact that over the Antarctica sea ice there does not have many water ponds and the surface moisture rise is not significant due to the quick melting of sea ice.

Fig. 4 Time series of monthly average extents of Antarctic sea ice in 2011 determined by RA-2 and microwave radiometers. The dash line is the difference value of RA-2-NSIDC.

Table 1 Statistic of the sea ice extent by RA-2 and NSIDC (unit: million of km2)

| Month Resource | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 8 | 9 | 10 |  | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA-2 | 9.10 | 3.97 | 4.42 | 6.77 | 10.21 | 13.40 | 16.44 |  | 18.23 | 19.11 | 19.00 |  | 17.07 | 14.07 |
| NSIDC | 4.68 | 2.47 | 3.61 | 6.74 | 10.82 | 13.76 | 16.28 |  | 18.09 | 18.90 | 18.48 |  | 16.15 | 12.20 |
| RA2-NSIDC | 4.42 | 1.50 | 0.81 | 0.03 | -0.61 | -0.36 | 0.16 | 0.14 |  | 0.21 | 0.52 | 0.92 |  | 1.87 |

Fig. 5 Time series of monthly average values of backscatter coefficients over two polar sea areas.

# 4 Conclusions

In this paper, the backscatter coefficient  derived from an active microwave senior RA-2, a radar altimeter carried by ENVISAT satellite, was used to research the sea ice distribution features. Compared with the NSIDC sea ice data, RA-2 presents a powerful ability to detect monthly boundaries of the sea ice. Over the two polar areas the boundaries of the sea ice are closely consistent with the NSIDC data. But during the summer when the sea ice melts quickly there exist some larger differences. In order to evaluate the accuracy of the altimetry method we calculated monthly extents of the sea ice. Because there has data gap in the central Arctic, we only evaluate the Antarctic sea ice extents. The altimetry result closely agrees with the NSIDC data in the whole year except the southern hemisphere summer, which mainly caused by the discrete thin ice and the quickly melting speed. The average difference of the sea ice extent between the altimetry and NSIDC result is 0.80Mkm2 and the standard deviation of the difference is 1.35Mkm2. But when excluding the summer the average of difference is just 0.20Mkm2 and the standard deviation is 0.50Mkm2. The accuracy tends to be best within the consolidated ice pack where the sea ice is relatively thick and ice concentration is high. The average difference in southern hemisphere winter (from June to September) is only 0.17 Mkm2. In summer the radar altimetry presents higher values because it has the relatively small footprint and is more sensitive to the discrete ice than radiometers. In addition, the moisture changes of the sea ice surface can also be illustrated by  which is sensitive to the moisture of reflected surface. It reveals that the sea ice is wetter over Arctic than Antarctic oceans in the warm season and the sea ice is drier in over Arctic than Antarctic oceans in the cold season.

In the future research, obtaining the sea ice heights by satellite radar altimeters will be an interesting and challenging subject which could provide the variations of the sea ice volume caused by the climate change. Although the ENVISAT was already retired, the new altimeters, i.e., Chinese HY-2 and India–French Saral/AltiKa, can still provide active microwave measurements over oceans of high latitude where sea ice covers.

Acknowledgements

This work was partially supported by the National Basic Research Program of China (973 Program) (Nos. 2012CB957704). The authors would like to thank ESA for providing ENVISAT RA-2 GDR and SGDR data. We thank Dr. Sun Yu at Delft University and Professor Chen Yongqi at The Hong Kong Polytechnic University of Technology for their helps and support.

Corresponding Author:

Yang Lei,

First Institute of Oceanography, State Oceanic Administration, Qingdao, China, 266061;

Tel: +86 0532-88965357;

E-mail: leiyang@fio.org.cn

Author biography:

Lin Zhu received her BS in 2012 from Shandong University of Science and Technology, China, in remote sensing science and technology. She is currently a second year graduate student majored in remote science. She conducts research on the quantitative remote sensing.

Lei Yang is an assistant researcher of First Institute of Oceanography, State Oceanic Administration (SOA), People’s Republic of China. His research interest is the application of satellite radar altimetry. He got his MS in geodesy at Shandong University of Science and Technology in 2013.

Dong-xu Zhou got his MS in Institute of Geodesy and Geophysics, Chinese Academy of Sciences in 2011. He is currently a second year doctoral student majored in marine surveying and charting at Geometrics college, Shandong University of Science and Technology, Qingdao, China. He conducts research on the sea level change based on GNSS and tide technology.

Xing-hua Zhou is a professor of The First Institute of Oceanography, SOA, People’s Republic of China. His research interest is the marine geodesy. He received his BS in geodesy from Wuhan University in 1985, his MS in surveying engineering from Wuhan University in 1995 and his PhD in marine science from Hong Kong Polytechnic University in 2003.

Qiu-hua Tang is a professor of The Institute of Oceanography, SOA. His research interest is the marine geodesy.

# References

- MASLOWSKI W, CLEMENT KINNEY J, HIGGINS M, et al. The Future of Arctic Sea Ice[J]. Annual Review of Earth and Planetary Sciences, 2012, 40: 625-654.

- WILLIAM K. Abrupt Mid-Twentieth-Century Decline in Antarctic Sea-Ice Extent from Whaling Records[J]. Nature, 1997, 389(6646): 57-60.

- CAVALIERI D J, GLOERSEN P, PARKINSON C L, et al. Observed Hemispheric Asymmetry in Global Sea Ice Changes[J]. Science, 1997, 278(5340): 1104-1106.

- PEROVICH D K and RICHTER-MENGE J A. Loss of Sea Ice in the Arctic[J]. Annual Review of Marine Science, 2009, 1: 417-441.

- BOE J, HALL A, and QU X. September Sea-Ice Cover in the Arctic Ocean Projected to Vanish by 2100[J]. Nature Geosci, 2009, 2(5): 341-343.

- SPREEN G, KALESCHKE L, and HEYGSTER G. Sea Ice Remote Sensing Using Amsr-E 89-Ghz Channels[J]. J. Geophys. Res, 2008, 113(C2): C02.

- ZWALLY H J, COMISO J C, PARKINSON C L, et al. Variability of Antarctic Sea Ice 1979–1998[J]. Journal of Geophysical Research: Oceans, 2002, 107(C5): 9-1-9-19.

- FU L-L, CHELTON D B, LE TRAON P-Y, et al. Eddy Dynamics from Satellite Altimetry[J]. Oceanography, 2010, 23(4): 14-25.

- CHELTON D B, GAUBE P, SCHLAX M G, et al. The Influence of Nonlinear Mesoscale Eddies on near-Surface Oceanic Chlorophyll[J]. Science, 2011, 334(6054): 328-332.

- DWYER R and GODIN R, Determining Sea-Ice Boundaries and Ice Roughness Using Geos-3 Altimeter Data, 1980, National Aeronautics and Space Administration: Washington, DC. p. 44.

- RAPLEY C G. First Observations of the Interaction of Ocean Swell with Sea Ice Using Satellite Radar Altimeter Data[J]. Nature, 1984, 307(5947): 150-152.

- ULANDER L and CARLSTROM A. Radar Backscatter Signatures of Baltic Sea Ice. in Geoscience and Remote Sensing Symposium, 1991. IGARSS'91. Remote Sensing: Global Monitoring for Earth Management., International. 1991. IEEE.

- DRINKWATER M R. K U Band Airborne Radar Altimeter Observations of Marginal Sea Ice During the 1984 Marginal Ice Zone Experiment[J]. Journal of Geophysical Research: Oceans, 1991, 96(C3): 4555-4572.

- CHASE J R and HOLYER R J. Estimation of Sea Ice Type and Concentration by Linear Unmixing of Geosat Altimeter Waveforms[J]. J. Geophys. Res., 1990, 95(C10): 18015-18025.

- YANG Y, DONGCHEN E, WANG H, et al. Sea Ice Concentration over the Antarctic Ocean from Satellite Pulse Altimetry[J]. Science China Earth Sciences, 2011, 54(1): 113-118.

- KOURAEV A V, PAPA F, BUHARIZIN P I, et al. Ice Cover Variability in the Caspian and Aral Seas from Active and Passive Microwave Satellite Data[J]. Polar Research, 2003, 22(1): 43-50.

- WEISSLING B and ACKLEY S. Antarctic Sea-Ice Altimetry: Scale and Resolution Effects on Derived Ice Thickness Distribution[J]. Annals of Glaciology, 2011, 52(57): 225-232.

- CONNOR L N, LAXON S W, RIDOUT A L, et al. Comparison of Envisat Radar and Airborne Laser Altimeter Measurements over Arctic Sea Ice[J]. Remote sensing of environment, 2009, 113(3): 563-570.

- LAXON S, PEACOCK N, and SMITH D. High Interannual Variability of Sea Ice Thickness in the Arctic Region[J]. Nature, 2003, 425(6961): 947-950.

- GUO J, YANG L, LIU X, et al. Decadal Variation of Surface Characteristics over Xinjiang, Western China, from Backscatter Coefficients of T/P Altimetry: Evidence of Climate Change[J]. Terrestrial, Atmospheric and Oceanic Sciences, 2013, 24(4): 565-579.

- GUO J Y, YANG L, LIU X, et al. On Temporal-Spatial Distribution of Backscatter Coefficients over China Determined by Topex/Poseidon Mission[J]. Science China-Earth Sciences, 2012, 55(12): 2068-2083.

- ESA, Envisat Ra2/Mwr Product Handbook, 2007.

- PAPA F, LEGR SY B, and R MY F. Use of the Topex-Poseidon Dual-Frequency Radar Altimeter over Land Surfaces[J]. Remote sensing of environment, 2003, 87(2-3): 136-147.

- FETTERER F M, DRINKWATER M R, JEZEK K C, et al., Sea Ice Altimetry, in Microwave Remote Sensing of Sea Ice, F D CARSEY, Editor 1992, American Geophysical Union: Washington, DC. p. 111-136.

7/19/2014