DISTRIBUTION AND GEOCHEMICAL CONDITIONS OF HEAVY METALS IN SOILS IN AROUND ALMALYK MINING AND INDUSTRIAL AREA

Bekmukhamedova Munisa Khamitovna, Sharipov Shavkat Mukhamajanovich

University of Geological Sciences, Tashkent, Uzbekistan

National University of Uzbekistan, Tashkent, Uzbekistan

bekmuxamedovamunisa@gmail.com

Abstract: In accordance with the internationally recognized "Concept of Sustainable Development," special attention is paid to the need to begin a global solution to environmental problems at the local, national and regional levels with environmental protection and rational use of natural resources. Technological progress has exceeded the rate of use of natural resources and pollution of the environment by pollutants. As a result, the natural balance is disrupted. The expansion of the activities of mining enterprises leads to the pollution of atmospheric air, soil and drinking water. In this regard, the need to carry out geoecological research in these areas shows the urgency of the work.

[Bekmukhamedova Munisa Khamitovna, Sharipov Shavkat Mukhamajanovich. DISTRIBUTION AND GEOCHEMICAL CONDITIONS OF HEAVY METALS IN SOILS IN AROUND ALMALYK MINING AND INDUSTRIAL AREA . N Y Sci J 2022;15(9):60-65] ISSN 1554-0200(print);ISSN 2375-723X (online) http://www.sciencepub.net/newyork. 08. doi:10.7537/marsnys150922.08.

Keywords: mining, Geosystem, soil, heavy metals, pollution, environment.

- Introduction

Solving environmental problems in ensuring sustainable development of industrial areas is one of the urgent issues of today. This is especially true for solid waste that accumulates in mining districts, that is, in areas where mineral extraction and processing enterprises are located. In areas where the mining industry is developed, environmental change is associated with surface mining, where 2/3 of the annual amount of mined rock is discarded as tailings at metal processing plants, and much of it is not used. As a result, there is pollution of geosystems with chemical elements, exceeding the maximum allowable concentration of harmful components several times [7].

- Analysis and results

Soils are an integral part of the natural environment and embody the information about natural and chemical changes of the environment that occurred throughout the period of technogenesis. They require detailed ecological and geochemical studies and constant observation, unlike any other component of the natural environment. In the areas where various industrial enterprises, especially chemical and metallurgical enterprises are located and directly adjacent to them, man-made changes often occur, heavy metals exceed the permissible limit, pollution, erosion and deflation of the soil layer are observed due to the dissolution of heavy metal compounds in the soil under the influence of acid rain. Restoration of the soil layer, which has been subjected to such a man-made impact, requires huge expenditures of labor and funds.

Open tailings ponds of ore processing plants, enterprises of metallurgical, chemical and other industries in the territory of mining areas have a negative impact on the ecological state of the environment.

Mineralogical and geochemical studies conducted by geochemists show that the amount of harmful metals in the soil samples is at a level slightly higher than the maximum allowable concentration. In such conditions, it is necessary to determine the laws of distribution of heavy metals throughout the basin, as well as to study and analyze the balance mechanism of the geosystem and its biological components.

An increase in the amount of heavy metals in geosystems can be harmful and even dangerous for living organisms. Infiltration of heavy metals into the soil occurs both naturally (weathering of rocks and minerals, erosion processes, volcanic activity) and technogenic (mining and processing of ores, fuel combustion, under the influence of vehicles, agriculture, etc.) way.

Indicators of soil pollution with heavy metals include the amount of metals in its content, the nature of metals, their chemical and harmful properties, the form of chemical compounds, etc.

Soil stability to pollution depends on its granulometric composition, amount of organic matter, acid-alkaline and oxidizing-reducing conditions, activity of microbiological and biochemical processes, etc. Soil, unlike other components of the natural environment, not only accumulates in itself harmful components, but also acts as a natural shield, controlling the migration of chemical elements in the atmosphere, hydrosphere and living organisms. The high content of heavy metals in the soil not only harms living organisms and plants in it, but also poses a threat to human health through food products grown in contaminated soils.

Heavy metals and their compounds that have fallen into the soil accumulate, disperse or pass into other forms depending on the characteristics of this geosystem, the characteristics of geochemical barriers. When heavy metals are integrally associated with the soil component, their negative impact on the soil and environment is negligible. In the conditions where the natural balance in the soil has changed, it becomes possible for heavy metals to become a soil solution, and the level of their impact on plants and living organisms through the soil increases dramatically [1].

In the conditions where the natural balance in the soil has changed, it becomes possible for heavy metals to become a soil solution, and the level of their impact on plants and living organisms through the soil increases dramatically. The degree of contamination of soils and plants depends on: the form of chemical compounds of elements in the soil; the presence of elements that form complex compounds with heavy metals and products and have the opposite effect on their effects; the processes of adsorption and desorption; the number of metallochemical forms corresponding to the soil and soil-climatic conditions.

It is known that the high content of lead, zinc, copper, cadmium, molybdenum, tungsten and other heavy metals has a significant impact on the geochemical conditions and environmental situation of this place. In this case, the degree of harmful effects of the elements depends on the forms of their occurrence. Mobile, i.e. quickly soluble in water, various compounds of toxic elements (salts, acids, etc.), as well as their ionic forms are quite active, and even their small amount poses a great danger to a living organism. Its forms in the form of hardly soluble compounds (minerals, oxides, etc.), on the contrary, pose little risk. Therefore, studying the forms of their occurrence combined with determining the amount of toxic elements in soil, water, plants, and organisms in areas exposed to technogenic impact is one of the main indicators of assessing the ecological situation.

Our research object is located in the middle reaches of the Akhangaran River basin, the largest industrial area of our republic. Large enterprises of gold, non-ferrous metals, coal mining, construction industry products and electricity production operate here. Multisectoral agriculture, consisting of cereals, melons, fruits and vegetables, etc., also occupies an important place.

N.E. Shukurov (2013) conducted mineralogical and geochemical study of soil samples taken from the impact zone of Almalyk mining and smelting complex in order to identify heavy metals in soils subjected to technogenic impact. As a result of research conducted on the basis of separation of soil samples into separate fractions and the use of the latest modern analytical equipment, the amount and forms of their occurrence of heavy metals were determined [3].

The following figure shows soil sampling sites around Almalyk mining and industrial area (Fig.1).

Fig.1. Soil sampling sites

The obtained soil samples were analyzed using modern analytical methods. The results showed that the content of heavy metals always increases with proximity to the source of pollution and decreases with distance from it. The table below shows the distribution of heavy metals in the soils of Almalyk mining region (Table 1).

Table 1. Distribution of heavy metals in soils of Almalyk mining district

| Sample number |  | Sc | V | Cr | Co | Ni | Cu | Zn | Ga | Rb | Sr | Y | Zr | Nb | Ba | Pb | Th | U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XIV/1 | Profile I-I | 24 | 149 | 43 | 13 | 16 | 1634 | 781 | 14 | 164 | 169 | 20 | 87 | 8 | 714 | 395 | 12.1 | 3.6 |
| IX/1 |  | 18 | 120 | 28 | 13 | 20 | 1519 | 229 | 14 | 178 | 202 | 22 | 123 | 12 | 676 | 115 | 14.1 | 4.7 |
| V/1 |  | 14 | 94 | 57 | 14 | 29 | 87 | 138 | 18 | 156 | 162 | 31 | 219 | 19 | 703 | 57 | 19.1 | 3.8 |
| I/1 |  | 17 | 108 | 82 | 14 | 37 | 129 | 223 | 14 | 109 | 184 | 26 | 166 | 14 | 610 | 72 | 11.5 | 2.4 |
| XIV/1 | Profile II-II | 24 | 149 | 43 | 13 | 16 | 1634 | 781 | 14 | 164 | 169 | 20 | 87 | 8 | 714 | 395 | 12.1 | 3.6 |
| XI/9 |  | 12 | 90 | 58 | 13 | 28 | 125 | 138 | 17 | 159 | 154 | 32 | 196 | 18 | 698 | 60 | 18.7 | 5.5 |
| VIII/9 |  | 11 | 78 | 46 | 10 | 22 | 83 | 111 | 17 | 157 | 183 | 29 | 197 | 18 | 719 | 52 | 18.2 | 3.5 |
| VI/10 |  | 11 | 80 | 47 | 11 | 23 | 77 | 132 | 16 | 158 | 170 | 29 | 197 | 18 | 712 | 54 | 18.7 | 4.5 |
| II/11 |  | 18 | 107 | 92 | 15 | 37 | 53 | 99 | 12 | 96 | 197 | 25 | 210 | 13 | 557 | 28 | 11.8 | 2.3 |
| XIV/1 | Profile III-III | 24 | 149 | 43 | 13 | 16 | 1634 | 781 | 14 | 164 | 169 | 20 | 87 | 8 | 714 | 395 | 12.1 | 3.6 |
| X/12 |  | 13 | 92 | 63 | 14 | 30 | 66 | 117 | 18 | 154 | 152 | 33 | 191 | 18 | 718 | 57 | 16.8 | 5.1 |
| XVI/10 |  | 13 | 88 | 56 | 13 | 26 | 143 | 222 | 17 | 159 | 167 | 31 | 198 | 19 | 766 | 104 | 19.1 | 4.1 |
| XIV/12 |  | 7 | 71 | 24 | 7 | 8 | 18 | 59 | 15 | 168 | 178 | 26 | 289 | 18 | 738 | 33 | 18.5 | 3.4 |
| XII/12 |  | 8 | 77 | 41 | 10 | 20 | 48 | 104 | 18 | 167 | 172 | 28 | 198 | 18 | 756 | 49 | 18.7 | 2.9 |
| XIV/1 | Profile IV-IV | 24 | 149 | 43 | 13 | 16 | 1634 | 781 | 14 | 164 | 169 | 20 | 87 | 8 | 714 | 395 | 12.1 | 3.6 |
| XIII/2 |  | 15 | 98 | 95 | 17 | 36 | 2946 | 1599 | 3 | 92 | 237 | 28 | 151 | 10 | 667 | 573 | 9.9 | 4 |
| X/3 |  | 14 | 114 | 85 | 14 | 45 | 195 | 644 | 16 | 127 | 201 | 31 | 174 | 14 | 847 | 218 | 12.6 | 4.7 |
| VII/4 |  | 11 | 89 | 56 | 10 | 21 | 187 | 664 | 15 | 116 | 212 | 22 | 173 | 12 | 792 | 367 | 9.1 | 4.8 |
| IV/5 |  | 14 | 83 | 53 | 12 | 24 | 108 | 776 | 14 | 115 | 218 | 23 | 183 | 14 | 680 | 522 | 11.8 | 4 |
| II/6 |  | 15 | 98 | 78 | 11 | 31 | 51 | 154 | 14 | 103 | 183 | 25 | 197 | 14 | 595 | 43 | 12.1 | 2.5 |
| XIV/1 | Profile V-V | 24 | 149 | 43 | 13 | 16 | 1634 | 781 | 14 | 164 | 169 | 20 | 87 | 8 | 714 | 395 | 12.1 | 3.6 |
| XIII/3 |  | 17 | 95 | 81 | 14 | 38 | 3137 | 1855 | 3 | 86 | 237 | 27 | 151 | 11 | 627 | 629 | 10 | 4.6 |
| XII/4 |  | 13 | 93 | 60 | 13 | 29 | 127 | 684 | 16 | 134 | 202 | 25 | 187 | 13 | 813 | 115 | 14.1 | 3.3 |
| XI/5 |  | 14 | 104 | 83 | 14 | 39 | 84 | 328 | 15 | 113 | 200 | 26 | 185 | 13 | 690 | 113 | 12.5 | 2.1 |
| VIII/7 |  | 17 | 109 | 89 | 18 | 41 | 46 | 119 | 13 | 94 | 219 | 24 | 187 | 13 | 572 | 34 | 11 | 2.8 |

Based on the above table, the results were analyzed and mapped using ArcGIS software.

Fig.2. Distribution of the element scandium along the Gejigen network

As can be seen from the figure, the element scandium is distributed in large quantities around the Koriz channel Near this area is Almalyk mining and smelting complex. Almalyk mining and smelting complex (AGMK) with its numerous mining facilities ( open pits, dumps, tailings ponds, mud accumulators) is the main source of air and soil pollution by heavy metals in Almalyk industrial area.

Anthropogenic sources of scandium (Sc) in the environment are various branches of industrial production. Rare earth elements and associated scandium wastes enter the air environment of industrial buildings at various stages of the technological process of their production and processing, as well as during their industrial application: during loading and unloading from reactors, filters, furnaces, as a result of operation of evaporation tanks, extractors, electrolyzers, certain types of devices. Scandium was first detected in the human body in 1935 by L.P. Vinogradov. The main way of entry of scandium into the human body is breathing; the distribution of various rare earth elements in the tissues of the body is not the same. Lighter ones accumulate in the liver in significant quantities, and heavier ones accumulate in bone tissue. Some of the rare earth elements remain in the lungs and lead to desquamative pneumonia, the formation of granulomatous nodules, and later the development of inflammatory phenomena in the peribronchial zones of the lungs [8].

- Conclusions

In short, the impact of heavy metals on soil ecology depends on their mobility and solubility. Clayey soils with high adsorption capacity and soils rich in organic matter (especially in the top soil layers) can retain these elements. This property is typical for soils with carbonate and neutral reaction. The content of harmful compounds in these soils is much lower than in loamy soils with acidic properties. Heavy metals trapped in the colloidal and organic parts of the soil limit biological activities.

References

- Bekmukhamedova M.Kh. (2017) Ohangaron havzasi o‘rta qismi geotizimlarining geoximik sharoiti va uni aholi salomatligiga ta’siri [Geochemical conditions of geosystems in the middle part of the Akhangaran basin and their impact on public health]. (Thesis written for the Master's academic degree). Tashkent. (in Uzbek).

- Lozanovskaya I.N., Orlova D.S., Sadovnikova L.K. (1998) Ekologiia i okhrana biosfery pri khimicheskom zagraznenii [Ecology and protection of the biosphere in chemical pollution]. Moscow: Vysshaya Shkola, 288 p. (in Russian).

- Miraslanov M.M., Zakirov M.M. (2003) Sostoianie ustoichivosti bortov Angrenskogo, Kurgashinkanskogo, Kalmakyrskogo, Sarychekinskogo glubokikh  karerov [State of the stability of Angren, Kurgashinkan, Kalmakkyr and Sarycheku deep pits]. Materialy Mezhdunarodnogo Simpoziuma «Geologicheskii risk: otsenka i umenshenie» [Materials of the International Symposium: "Geological risk: assessment and reduction"], Tashkent, pp. 90-96. (in Russian).

- Rafikov V.A. (2014) Tabiat va jamiyat munosabatlarini optimallashtirishning ekologik-geografik asoslari [Ecological and geographical bases of optimization of relations between nature and society]. Tashkent: "Munis design group".  (in Uzbek).

- Shukurov N.E. (2013) Raspredelenie tiazhelykh metallov v pochvakh Almalykskogo gornopromyshlennogo raiona i ikh vliianie na pochvennye mikroorganizmy [Distribution of heavy metals in soils of Almalyk mining district and their impact on soil microorganisms]. Ecological Bulletin, vol. 4, no 4, pp.  26-31.

- Shukurov N.E. (2001) Tog‘-konchilik va metallurgiya sanoatining atrof-muhitga ta’siri [Impact of mining and metallurgical industry on the environment]. Ecological Bulletin, vol. 3, pp.  5-7.

- Shukurov Sh. (2011) Antropogen jarayonlarning Ohangaron daryosi vodiysi geoekologik xolatiga ta’sirini baholash [Assessment of the impact of anthropogenic processes on the geoecological condition of the Akhangaran river valley]. (Master's thesis). Tashkent. (in Uzbek).

- https://studexpo.net/784601/himiya/vozdeystvie_berilliya_skandiya_zhivye_organizmy.

9/21/2022