Physio-Chemical analysis of ground water of selected areas of Mysore City, Karnataka, India

B.Nirmala1, N.J.Ranjitha2

1. Assistant Professor, Dept. of Chemistry, University College of Science, Tumkur-572 103,Karnataka State, India.

2. Lecturer, Dept. of Chemistry, JSS College for Women (Autonomous), Mysore-570 008,Karnataka state, India.

nirmala2528@gmail.com

Abstract: A laboratory study was conducted to monitor the ground water quality of selected sites of Mysore city by examining the various physico-chemical parameters like pH, TDS, DO, total hardness, COD, etc. Ground water samples were collected from various locations in study area during monsoon and post monsoon season. The results are analyzed comparatively and conclusions regarding the suitability of the use of such waters are made.

[Dr. B. Nirmala, N.J.Ranjitha, Physico-Chemical analysis of ground water of selected areas of Mysore City, Karnataka, India. World Rural Observations 2011;3(3):88-91]; ISSN: 1944-6543 (Print); ISSN: 1944-6551 (Online). http://www.sciencepub.net/rural.

Key words: Ground water, water quality, physico-chemical parameters, COD, DO, TDS.

1. Introduction:

Water is extremely essential for survival of all living organisms. The quality of water is of vital concern for mankind since it is directly linked with human health, protection of the environment and sustainable development. Increasing population and its necessities have led to the deterioration of surface and sub-surface water. Ground water is the major source of drinking water in both urban and rural areas. The domestic sewage and industrial waste are the leading causes of ground water pollution (Dahiya S.D,1999; Saharan,2009). Consequently number of cases of water borne diseases has seen which is a cause of health hazards (Desai, 1995; Naik, 2005; Grahow, 2001). So basic monitoring on water quality has been necessitated to observe the demand and pollution level of ground water (WHO, 1999). A good number of water analysis experiments are regularly conducted by different groups of chemists and biologists across the country (Nath, 2001; Kalkar, 2001; Karlo, 1992; Vijender singh, 2006).

The present work is an attempt to carry out qualitative analysis of some physico-chemical parameters of ground water during monsoon and post monsoon season in and around Mysore city.

2. Materials and Methods:

2.1. Study area: Mysore is one of the famous tourist centers of Asia, located between latitude 11o45’ to 12o40’ N and longitude 75o57’ to 77o15’ E. The district covers an area of about 6940 sq.km and is accessible from several parts of the world.

2.2. Sample collection: 150 ground water samples from four different zones (North, South, East and West) of Mysore city were collected during Monsoon and Post monsoon seasons of 2009-2010. The water was pumped out from bore wells a few minutes before sample collection. Temperature and pH were determined immediately at the sampling station.

3. Physico-chemical analysis of ground water:

The collected samples were analyzed on various physico-chemical parameters such as dissolved oxygen, TDS, total hardness, calcium hardness, magnesium hardness, chloride, iron, COD as per standard methods of analysis of water and waste water (APHA) and the results were compared with the Indian standards for potable water.

The average values of various physico-chemical parameters during monsoon and post monsoon seasons are presented in Table 1 and 2. A comparison of ground water quality of the study areas with drinking water standards (WHO) are presented in table 3 and 4. The data of the chemical results showing consider variation which reflects their chemical composition.

A brief account of the range of these parameters is also discussed.

pH:  The average values of water samples are alkaline ranging from 7.01 to 8.4 during monsoon and 7.31 to 8.4 during post monsoon. According to Kannan(1991) pH between 6.7 and 8.4 is suitable while pH below 5.0 and above 8.0 are detrimental.

TDS: TDS of ground water samples are due to vegetable decay, evaporation, disposed effluents and chemical weathering of rocks. TDS values varied from 0.02 to 1.043mg/l during monsoon and 0.01 to 0.08 during post monsoon.

EC: Electrical conductivity is a measure of water capacity to convey electric current. It signifies the amount of total dissolved salts (Kaur, 1999)

Total Alkalinity: Alkalinity of samples is in between 95 to 490 ppm during monsoon and 117 to 415 ppm during post monsoon season which indicates that water from all selected areas are hard. Higher values of alkalinity registered during monsoon which may be due to presence of excess of free CO2. Alkalinity itself is not harmful to human beings.

Total hardness: Hardness is the property of water which prevents the lather formation with soap and increases the boiling points of water (Goel,1986). Hardness of water mainly depends upon the amount of calcium or magnesium salts or both. The hardness values ranges from 145 to 655mg/l during monsoon and 165 to 480mg/l during post monsoon season.

Cl-: The chloride concentration serves as an indicator of pollution by sewage. People accoustomed to higher chloride in water are subjected to laxative effects (Kaur, 1999).  The values range from 120 to 210 mg/l during monsoon and 40 to 210 mg/l during post monsoon season.

Ca2+: Calcium and magnesium are directly related to hardness. Calcium concentration ranges between 45 to 188 mg/l during monsoon and 50 to 230 during post monsoon season.

Dissolved Oxygen: Dissolved oxygen is important parameter in water quality assessment and biological processes prevailing in the water. The DO values indicate the degree of pollution in the water bodies. Dissolved oxygen values varied from1.2 to 7.6 mg/l during monsoon and 4.8 to 8.0 mg/l during post monsoon season.

COD and BOD:  All samples from monsoon and post monsoon season analyzed for COD and BOD. The values observed are in between 0 to 8.

TABLE 1: Average values of Physico Chemical parameters of ground water of Mysore City, Monsoon season (2009-2010)

| Sl No | Area | pH | EC | TDS | TH | Ca2+ | TA | Cl- | DO | COD | BOD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NORTH |  |  |  |  |  |  |  |  |  |  |
| 1 | Bannimantap | 7.06 | 0 | 0.116 | 532 | 188 | 341 | 126 | 5.4 | 0.02 | 2.8 |
| 2 | Mandimohalla | 7.6 | 0.7 | 0.026 | 460 | 120 | 375 | 120 | 1.6 | 0.096 | 0.4 |
| 3 | N.R.Mohalla | 7.5 | 0.7 | 0.029 | 640 | 135 | 340 | 160 | 6.8 | 0.08 | 5.2 |
|  | SOUTH |  |  |  |  |  |  |  |  |  |  |
| 4 | Siddappa square | 7.22 | 0 | 0.177 | 375 | 100 | 220 | 120 | 4.0 | 0.016 | 0.4 |
| 5 | J P Nagar | 8.84 | 0.8 | 0.047 | 655 | 135 | 415 | 200 | 1.2 | 0.092 | 0.8 |
| 6 | Jayanagar | 7.01 | 0.7 | 0.027 | 445 | 135 | 390 | 130 | 0.8 | 0.008 | 2.1 |
|  | EAST |  |  |  |  |  |  |  |  |  |  |
| 7 | Saraswathipuram | 7.81 | 0.7 | 0.027 | 350 | 105 | 240 | 140 | 6.0 | 0.012 | 4.8 |
| 8 | TK Layout | 7.41 | 0.3 | 0.012 | 145 | 45 | 95 | 410 | 7.6 | 0.016 | 6.8 |
| 9 | Sharadadevinagar | 7.06 | 0.4 | 1.043 | 210 | 100 | 150 | 130 | 5.2 | 0.032 | 4.4 |
|  | WEST |  |  |  |  |  |  |  |  |  |  |
| 10 | Nazarbad | 7.52 | 0.9 | 0.045 | 650 | 75 | 490 | 160 | 5.6 | 0.032 | 1.2 |
| 11 | Siddarathalayout | 7.78 | 0.8 | 0.02 | 340 | 120 | 290 | 120 | 4.8 | 0.0016 | 0.9 |
| 12 | Kyathamaranahalli | 7.14 | 0.6 | 0.038 | 600 | 130 | 490 | 120 | 2.0 | 0.0016 | 0.6 |

Note: All parameters are expressed in mg/l except pH and conductivity (µs/cm)

TABLE - 2 Average values of Physico Chemical parameters of ground water of Mysore City, Post Monsoon season (2009-2010)

| Sl No | Area | pH | EC | TDS | TH | Ca2+ | TA | Cl- | DO | COD | BOD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NORTH |  |  |  |  |  |  |  |  |  |  |
| 1 | Bannimantap | 7.31 | 1.21 | 0.02 | 360 | 230 | 350 | 210 | 6.0 | 0.01 | 2.8 |
| 2 | Mandimohalla | 8.2 | 0.9 | 0.023 | 375 | 70 | 355 | 110 | 4.8 | 0.06 | 2.0 |
| 3 | N.R.Mohalla | 7.4 | 0.5 | 0.03 | 310 | 110 | 285 | 155 | 5.2 | 0.03 | 2.5 |
|  | SOUTH |  |  |  |  |  |  |  |  |  |  |
| 4 | Siddappa square | 8.4 | 0.8 | 0.02 | 360 | 110 | 250 | 90 | 6.4 | 0.01 | 2.0 |
| 5 | J P Nagar | 8.1 | 0.6 | 0.03 | 305 | 98 | 290 | 125 | 5.8 | 0.02 | 2.2 |
| 6 | Jayanagar | 8.0 | 1.0 | 0.03 | 420 | 200 | 350 | 110 | 6.4 | 0.03 | 2.8 |
|  | EAST |  |  |  |  |  |  |  |  |  |  |
| 7 | Saraswathipuram | 8.2 | 1.0 | 0.03 | 480 | 315 | 415 | 120 | 7.2 | 0.01 | 3.2 |
| 8 | TK Layout | 7.71 | 0.2 | 0.02 | 165 | 50 | 165 | 40 | 8.4 | 0.05 | 2.4 |
| 9 | Sharadadevinagar | 7.8 | 0.8 | 0.022 | 232 | 176 | 205 | 169 | 7.5 | 0.03 | 2.6 |
|  | WEST |  |  |  |  |  |  |  |  |  |  |
| 10 | Nazarbad | 7.5 | 0.5 | 0.01 | 248 | 135 | 117 | 97 | 6.8 | 0.02 | 2.4 |
| 11 | Siddarathalayout | 7.8 | 0.1 | 0.04 | 195 | 108 | 162 | 107 | 7.2 | 0.02 | 2.5 |
| 12 | Kyathamaranahalli | 7.71 | 0.2 | 0.08 | 130 | 90 | 155 | 110 | 7.6 | 0.03 | 2.8 |

Note: All parameters are expressed in mg/l except pH and conductivity (µs/cm)

Table 3.	Comparison of ground water quality at the study areas with drinking water areas during monsoon period (WHO 1993)

|  | Values from collected samples |  |  |  |
| --- | --- | --- | --- | --- |
| Parameters | Min | Max | Mean | WHO |
| pH | 7.01 | 8.4 | 7.705 | 7 – 8.5 |
| EC | 0.0 | 0.1 | 0.05 | 1400 |
| TDS | 0.02 | 1.043 | 0.531 | 500 |
| TH | 145 | 655 | 400 | 500 |
| TA | 95 | 490 | 292 | 120 |
| Ca2+ | 45 | 188 | 116 | 100 |
| Cl- | 120 | 210 | 165 | 250 |
| DO | 1.2 | 7.6 | 4.4 | - |
| BOD | 0.4 | 6.8 | 3.6 | 5.0 |
| COD | 0.016 | 0.08 | 0.096 | 4.0 |

Table 4. Comparison of ground water quality with drinking water standard at the study standard during post monsoon period (WHO 1993)

|  | Values from collected samples |  |  |  |
| --- | --- | --- | --- | --- |
| Parameters | Min | Max | Mean | WHO |
| pH | 7.31 | 8.4 | 7.85 | 7 – 8.5 |
| EC | 0.1 | 1.21 | 0.65 | 1400 |
| TDS | 0.01 | 0.08 | 0.045 | 500 |
| TH | 130 | 480 | 370 | 500 |
| TA | 117 | 415 | 324 | 120 |
| Ca2+ | 50 | 215 | 157 | 100 |
| Cl- | 40 | 210 | 145 | 250 |
| DO | 4.8 | 8.0 | 6.4 | - |
| BOD | 2.0 | 3.2 | 2.6 | 5.0 |
| COD | 0.01 | 0.06 | 0.04 | 4.0 |

4. Conclusions:

The various parameters studied are within the permissible limit as per WHO norms for drinking purposes in the studied period. Except few parameters like hardness and Chloride, Ca2+ were reported more than the permissible limit indicating the need of some treatment for minimization of the these parameters. The findings of the present work is also recommended that quality of ground water under study is nearly fit for drinking purpose and domestic use.

Acknowledgement:

The authors are grateful to UGC for financial support to carry out this work.

Author for all Correspondence:

Dr.B.Nirmala

Assistant  professor

Departement of Chemistry

University College of Science

Tumkur University,  Tumkur-572 103.

E-mail: nirmala2528@gmail.com

References:

- Garg V.K., Chaudhary A and Dahiya  S.D, An apparaisal of ground water quality in some village of district Jind district, Haruyana, Indian J.Environ.Prot. 1999; 19(4):267.

- Gupta D.P., Sunitha and Saharan J.P, Physio Chemical analysis of ground water of selected area of Kaithal City(Haryana),India, Researcher, 2009;1(2).

- Desai P.V, Water quality of Dudhsagar river at Dudhsagar(Goa), India, Poll.Res. 1995; (4), 377-382.

- Guide lines for drinking water quality, WHO, Geneva, 1999;2nd ed., 97-100

- Elizabeth K.M and Premnath Naik L, Poll Res., 2005;24(2), 337-340.

- Muller E.E, Ehlers M.M and Grabson ,The occurrence of E.coli 0157:H7 in south African water sources intended for direct and indirect human consumption, 2001; Water Research, 35, 3085-3088.

- Nath D, J Inland Fresh Soc. India, 2001; 33(2), 37-41

- Kelkar H.P.S et al, J.IWWA, 2001;Jan-Mar 39-43.

- Ramteke P.N, Battacharjee J.W and Karlo N J.Appl.Bact., 1992;j 72,352.

- Vijender Singh, Res. J.Chem and Env., 2006; 10(3), 62-66.

- APHA Standard methods for examination of waters and waste waters, 1995; 2-4, 29-179, American Public Health Association.

- Kannan K Fundamentals of Environmental Pollution, S. Chand and Co. Ltd, New Delhi, 1991.

- Sudhir Dahiya and Amarjeet Kaur, Assessment of physic-chemical characteristics of underground water in rural areas of tasham subdivisions, Bhiwani district, Haryan, J.Environ.Poll, 1999;6(4), 281.

- Trivedy R.K and Goel P.KChemical and Biological methods for water Pollution Studies, Environmental Publication, Karad; 1986.

- IS: 10500 Drinking water specification 1992 (reaffirmed 1993).

20/06/2011